# Week 3 Data Note: Reprojection, Clipping and Quality Checks

## 1. Coordinate system chosen and why

I chose WGS 84 / UTM Zone 32N (EPSG:32632). The original data was in a geographic coordinate system (degrees), which is not suitable for measuring distance. My project needs distances in metres (for example 2 km and 5 km reach zones around facilities), so I changed to a projected coordinate system that uses metres. Okura Ward falls within UTM Zone 32N.

## 2. What I reprojected and what I clipped

- Reprojected to EPSG:32632: health facilities, major highway (Highway1), minor roads (Highway2), and the Okura Ward boundary.

- Clipped to the Okura Ward boundary: health facilities (15 points), Highway1 (1 feature, about 14 km), and Highway2 (71 segments, about 171 km).

## 3. The five quality checks

| # | Check | Result |
| --- | --- | --- |
| 1 | Coordinate system | The original data was in a geographic CRS. All four layers in the final file are now in EPSG:32632. |
| 2 | Empty or null values | The key facility fields I need (name, type, ownership, functional status) have no missing values. For roads, many fields are empty (see section 4). |
| 3 | Duplicate features | No duplicate features in any layer. |
| 4 | Geometry validity | All geometries are valid, with no empty or broken geometries. |
| 5 | Coverage of study area | All facilities and roads fall inside the ward boundary. The roads span nearly the full east-west and north-south extent of the ward (about 358 sq km). |

## 4. Problems found and whether I fixed or flagged them

- Fixed: coordinate system (reprojected to metres).

- Flagged: missing road attributes. In Highway2, 71 of 71 segments have no name, 66 have no surface information, and 70 have no lane count. Road quality and surface type cannot be judged from this data.

- Flagged: non-road lines in the road layer. 7 of the 71 Highway2 segments are not roads: 4 streams, 1 river and 2 administrative boundary lines. They have no road type and should be excluded from the road analysis (the other 64 are roads).

- Flagged: facility functional status. 7 of the 15 facilities are marked “Unknown” for functional status, so I cannot confirm they are operating.

- Flagged: source data warnings. GRID3 marked 3 facilities with issues such as “outside of ward boundary” (one also “out of settlement extent”). After clipping, all 15 fall inside my ward boundary, but their positions should be treated with some caution.

- Flagged: empty fields. The settlement field is empty for all 15 facilities, and the alternative name is empty for 8.

## 5. Where the analysis-ready file lives

okura_ward_analysis.gpkg in my repository. It contains four layers: Health_facilities_Projected_Okuraward — clipped, Highway1_projected_Okura — clipped, Highway2_projected_Okura — clipped, and Okura_ward.