Month 1 Summary

1. My question

What is the spatial distribution of health facilities in Okura Ward, Dekina LGA, Kogi State, and how accessible are these facilities relative to the ward's existing road network?

2. Operation I ran and why

I ran a buffer of 2,000 m (2 km) around the 15 health facilities, using 20 segments, in a projected CRS (WGS 84 / UTM Zone 32N, EPSG:32632, so distances are in metres). I chose a buffer because it shows directly which parts of the ward lie within easy reach of a facility, and which do not. I chose 2 km because it is an easy walking distance (about 25 to 30 minutes at walking pace).

I then dissolved the buffers into one shape and clipped it to the Okura Ward boundary, so I could measure the share of the ward that is covered.

3. What I expected and what I got

What I expected (written before running the buffer):

- 15 buffer features, one for each health facility.

- Each 2 km circle to cover about 12.6 sq km (12,566,000 sq m).

- Some circles to overlap, because some facilities are close together.

- The zones to follow the roads, because most facilities sit along roads. Areas without roads would have no facility within 2 km.

- Together the zones to cover less than half of the ward. Note: my very first guess (about 70%) was made for a 5 km buffer. After I switched to 2 km and looked at the map, I revised it to about 30%.

What I got:

- 15 features in the buffer layer, matching my expectation.

- After dissolving and clipping to the ward, the zones cover 129,597,821 sq m (about 129.6 sq km). The ward is 357,931,534 sq m (about 357.9 sq km). So 36.2% of the ward is within 2 km of a health facility, and about 63.8% is not.

- The circles do overlap in the northwest, the east and the south.

How I checked the result:

- Map: the circles sit centered on the facilities and look the same size.

- Row count: 15 rows in the non-dissolved buffer layer, as expected.

- One feature by hand: I measured from one facility to the edge of its circle and got 1,963 m against the expected 2,000 m. The small difference comes from the circle being made of straight segments and from clicking by hand.

- Empty geometry: the expression is_empty_or_null($geometry) returned no matching features, so there are 0 empty geometries.

4. What surprised me

- The coverage was 36.2%, a bit higher than my 30% estimate, but still well under half the ward.

- Facilities do sit along roads, but some roads, especially in the east and across the south, still have no facility within 2 km. So being near a road does not mean being near a facility.

5. Data I still need

- Population or settlement data, so I can count how many people live inside and outside the 2 km zones, not just how much land.

- Travel distance along the roads (a network analysis). A buffer only measures straight-line distance, so real travel distances are longer.

- Road surface and condition. Most road attributes in my data are empty (no surface for 66 of 71 minor road segments).

- Functional status of the facilities. 7 of the 15 are marked "Unknown", so I cannot confirm they are operating.

- Cleaner road data. My minor roads layer includes 7 lines that are not roads (streams, a river and administrative boundaries).